Here is the complete architecture we built for your project. This is a very standard and professional Client-Server Architecture specifically designed for real-time AI computer vision:

1. The Frontend (Client / Browser)

- Technologies: HTML5, CSS3, JavaScript.

- Role: This is the dashboard you see in your browser.

- How it works: It uses your computer's webcam to capture video. JavaScript takes a "snapshot" of the video frame every 150 milliseconds (about 6-7 times a second), converts the image into a lightweight text string (Base64 format), and shoots it instantly to the server.

2. The Communication Layer (The Bridge)

- Technologies: WebSockets.

- Role: This is a two-way, always-open tunnel between your browser and the Python server.

- How it works: Instead of the browser constantly asking the server "Are you done yet?", the WebSocket allows the browser to rapidly fire images at the server, and the server can instantly push the results back down the split-second it finishes processing.

3. The Backend Server

- Technologies: Python, FastAPI, Uvicorn.

- Role: Receives the images from the browser and manages the connections.

- How it works: FastAPI listens on the WebSocket channel. When an image arrives, it converts the text format back into a readable image format (using numpy and cv2) and passes it to the AI.

4. The AI Inference Module (The Brain)

- Technologies: PyTorch (

.pth models), OpenCV (Haar Cascades), MediaPipe.

- Role: The heavy lifting. This is where your trained modelle_mask and modelle_gloves live.

- How it works:

  - Faces: It first uses OpenCV to scan the image to find where faces are located. Then it crops out the face and feeds it into your MobileNetV2 PyTorch model to see if it has a Mask.

  - Hands: At the same time, it uses Google MediaPipe to scan for hands/fingers. Once it maps the hand, it crops it and feeds it into your PyTorch Gloves model.

  - It calculates the coordinates (the X and Y location of the boxes) and the confidence scorces, packages them into a simple JSON dictionary, and hands it back to FastAPI.

The Loop: FastAPI shoots that JSON dictionary back down the WebSocket to your browser, and JavaScript draws the Green and Red boxes matching those coordinates over your video. This entire process happens over and over again multiple times per second!

Step 1: Start the Python Backend Open your terminal (cmd or PowerShell) in your project folder (C:\Users\nafed\OneDrive\Bureau\PFE\Integrate modele with website) and run:

bash

python -m uvicorn app:app --reload

Keep this terminal open. This turns your computer into a local server that runs the AI models.

Step 2: Open the Website Open your web browser (Chrome, Edge, etc.) and go to: http://127.0.0.1:8000

Step 3: Connect Your Camera On the website, click the blue "Start Camera" button and click "Allow" when the browser asks for permission to use your webcam. You should see yourself on the screen.

Step 4: Start AI Detection Click the "Start Inference" button. The website is now sending video frames to your Python backend to run through your models.

Step 5: Verify the Results

- Wearing Mask & Gloves: Safe (Green boxes will appear around your face and hands).

- Missing Mask or Gloves: Violation (Red boxes will appear and the dashboard status will alert you).